CIVE 580C8 – Project Progress Log

Non-Revenue Water Audit Agent (Option #5) Student: Jon Cronin Progress Summary Document Date: October 19, 2026

Section 1 — Summary of Prompts and Progress (Today)

1. Master Meter Synthetic Data Script Completed (generate_master_meter.py)

You finalized all design parameters for the master meter dataset, including:

- Flow units: GPM

- Cumulative volume: gallons + MG

- Meter accuracy: ±0.5%

- Resolution: 1-minute

- Full calendar year: 2026

- Realistic diurnal curves

- Seasonal variation

- Small storm-event spikes

- Deterministic seed

- Lab-style anomalies

- M36 Task 1 alignment

Based on these specifications, a complete, standard-library-only Python script was generated. The script:

- Produces 525,600 rows of synthetic master-meter data

- Includes outages on days 91 and 275

- Implements realistic flow behavior consistent with a single-pressure-zone system

- Tracks cumulative volume in both gallons and MG

- Introduces subtle operational noise and accuracy-driven apparent losses

- Writes output to data/master_meter_2026.csv

This script fully satisfies the requirements for Task 1 — Quantify Volume of Water Supplied.

2. Storage Tank Synthetic Data Script Completed (generate_storage_tank.py)

You defined the complete physical and operational characteristics of the system’s composite elevated tank:

- Composite elevated tank

- 0.5 MG total capacity

- 50-ft diameter

- 37-ft head range

- Normal operating band: 20–36 ft

- Fire/emergency storage: 0–20 ft

You selected:

- Operating Strategy A — classic daily fill-and-draw

- Pump Control Option A — pumps ON < 20 ft, OFF ≥ 36 ft

- Anomalies aligned with master-meter outage days

A full Python script was generated to simulate tank level behavior for the entire 2026 calendar year. The script includes:

- 1-minute resolution

- Deterministic seasonal and diurnal draw patterns

- Pump refill logic

- Fire/broken-main emergency draw events

- Sensor anomalies (dropout + flatline) matching master-meter outage days

- Realistic level cycling between 20–36 ft

- Occasional dips below 20 ft after adjusting emergency draw multiplier

The script writes output to data/storage_tank_2026.csv and supports M36 Tasks 1–4 by providing realistic operational storage behavior.

3. Fire-Flow Behavior Adjustment

After reviewing initial tank output, you noted that the tank did not dip below 20 ft during emergency events. You provided engineering calculations for realistic fire-flow demands:

- 1,000 gpm for 30 min → ~2 ft drop

- 1,000 gpm for 2 hours → ~8 ft drop

- 2,000 gpm for 2 hours → ~16 ft drop

Based on this, the emergency draw multiplier was increased from ×3 to ×6, producing realistic dips into the emergency storage zone while maintaining stable daily cycling.

4. Next Steps

With Scripts 1 and 2 complete, the next synthetic datasets to generate are:

- AMI customer meters (15-minute interval cumulative volume)

- AMR customer meters (monthly reads)

- Unbilled metered uses

- Unbilled unmetered uses

- Apparent losses

- System attributes

- Cost data

These will support Tasks 2–9 and complete the synthetic water balance.