CIVE 580C8 – Project Progress Log

Non-Revenue Water Audit Agent (Option #5)

Student: Jon Cronin

Project Progress Summary — Synthetic Water System Data

Date: October 14, 2026

1. Final Project Architecture Confirmed

We established the complete modular architecture for synthetic data generation, modeled after the structure used in Labs 1–7. Each dataset will be generated by its own standalone Python script, using only the standard library, fixed seeds, deterministic behavior, and realistic anomalies — exactly matching the course’s expectations.

Confirmed script list:

- generate_master_meter.py

- generate_storage_tank.py

- generate_customer_meters_AMI.py

- generate_customer_meters_AMR.py

- generate_unbilled_metered.py

- generate_unbilled_unmetered.py

- generate_apparent_losses.py

- generate_system_attributes.py

- generate_cost_data.py

This mirrors the lab folder structure and ensures reproducibility, clarity, and auditability.

2. Fictitious Water Distribution System Defined

You provided a complete description of the synthetic water system that will serve as the foundation for all data generation and M36 audit tasks. This ensures internal consistency across Tasks 1–4.

System Overview

- Serves 1,000 customers

  - 900 residential

  - 70 commercial

  - 30 light industrial

- Located in a fictional northern Colorado town

- Operates as a single pressure zone

- All water enters through one master meter at the treatment plant

- System includes one storage tank for peak shaving and operational storage

Customer Meter Inventory

- 1,000 total meters (Account IDs 00001–01000)

- 700 AMI meters

  - 650 residential

  - 35 commercial

  - 15 industrial

  - Report 15-minute cumulative volume

  - Data transmitted daily

- 300 AMR meters

  - Read monthly during the third week

  - Report total cumulative volume since last read

Demand Characteristics

Residential

- ADD: 100–200 gpd

- MDD: 250–500 gpd

- PHD: 350–700 gpd

- Most have irrigation systems

- Irrigation occurs late spring → early fall

Commercial / Light Industrial

- ADD: 100–1,000 gpd

- Some high-use customers (laundromats, breweries, car washes)

- Some have irrigation

Operational Notes

Synthetic data will reflect:

- Seasonal irrigation patterns

- Peak-hour demand behavior

- Random variability

- Storage tank cycling

- Master meter diurnal curves

- AMI/AMR reporting differences

- Realistic anomalies (missing data, spikes, outages)

3. Master Meter Dataset Requirements Finalized

We defined the specifications for the first synthetic dataset:

- Single master meter feeding the entire system

- 1-minute resolution

- Full calendar year of data (2026)

- Realistic diurnal flow curves

- Includes all relevant M36 Task 1 attributes:

  - timestamp

  - instantaneous flow

  - cumulative volume

  - meter status flags

  - meter accuracy class

  - outages / anomalies

  - calibration metadata (separate script but referenced)

Storage tank data will be generated separately in Script #2.

4. M36 Integration Confirmed

All synthetic data will be generated to support:

- M36 Chapter 1 (overview)

- M36 Chapter 2 (top-down method)

- Tasks 1–4 and all subtasks

This ensures the final audit behaves like a real utility’s water balance.

5. Water Loss Strategy Decided

We discussed two approaches:

- Pre-define a true water-loss profile

- Let water loss emerge randomly

We selected Option 1, because it produces a coherent, realistic, defensible system and aligns with M36 expectations.

Final Water-Loss Targets (Baseline for Synthetic System)

- NRW: 8–12%

- Apparent Losses: 1–3%

- Real Losses: 5–9%

- Authorized Unbilled: 0.5–2%

These values will guide how each dataset is generated so the final audit produces believable results.

6. Next Steps (Starting Tomorrow)

- Begin drafting generate_master_meter.py

- Produce synthetic 2026 master-meter data

- Test and inspect the output

- Move on to Script #2 (storage tank levels)

If you want, tomorrow we can start generating the first script immediately.